CSA1208 – COMPUTER ARCHITECTURE

ASSESSMENT TOOL 1 – CONSTRAINT-BASED PROBLEM SOLVING

# Complete Solutions – Annexure A

# Q1. Real-Time Embedded I/O Communication Mechanism

## Problem / Constraints

- CPU utilization must be below 15%.

- Multiple sensors generate interrupts every 1 ms.

- Vectored interrupt handling is required.

- Choose memory-mapped or I/O-mapped I/O for optimal latency.

## Proposed Solution

Use memory-mapped I/O with a vectored interrupt controller. Sensor registers are accessed using normal load/store instructions, while the interrupt controller identifies the source and transfers control directly to the appropriate ISR.

## Architecture

Sensors (S1, S2, S3, …)

↓

Vectored Interrupt Controller

↓

CPU

↓

Memory-Mapped I/O Registers

↓

Data Buffer

## Working

Each sensor generates an interrupt. The interrupt controller identifies the source and supplies its vector number. The CPU jumps directly to the corresponding ISR, reads the sensor register, places the data in a buffer, and returns to normal execution.

## CPU Utilization Example

If one ISR takes 10 µs and one sensor generates 1000 interrupts/s, the ISR consumes approximately 1% CPU time. For multiple sensors, the combined ISR time must remain below 15%; interrupt coalescing and efficient buffering can further reduce overhead.

## Memory-Mapped vs I/O-Mapped

Memory-mapped: shares the memory address space and uses normal load/store instructions. I/O-mapped: uses a separate I/O address space and special I/O instructions. Selection: memory-mapped I/O for this low-latency embedded design.

## Justification

Memory-mapped I/O provides simple and fast register access. Vectored interrupts reduce the time required to identify the interrupt source. Buffering and short ISRs reduce CPU overhead, helping maintain utilization below the 15% target.

# Q2. High-Speed NVMe I/O Subsystem

## Problem / Constraints

- Storage throughput must exceed 3 GB/s.

- NVMe over PCIe must be used.

- CPU should not handle bulk data transfer.

- DMA controller and interrupt notification must be integrated.

## Proposed Solution

Use an NVMe SSD connected through PCIe with DMA and MSI-X interrupt notification. The CPU submits commands, while the NVMe controller performs bulk data movement directly between the SSD and main memory. Architecture

CPU

↓ commands/descriptors

PCIe Root Complex

↓

PCIe Link

↓

NVMe Controller

↓

DMA Engine ↔ Main Memory

↑

MSI-X completion interrupt

## Working

The CPU places an NVMe command in the submission queue. The NVMe controller executes the command and uses DMA to transfer data directly to or from main memory. After completion, the controller generates an interrupt, allowing the CPU to process the completion entry.

## DMA Benefit

Without DMA: SSD → CPU → Memory. With DMA: SSD → DMA/controller → Memory. Therefore, CPU intervention is limited to command setup and completion processing.

## Throughput Target

The design must be implemented with a PCIe/NVMe configuration whose effective storage bandwidth is above the required 3 GB/s.

## Justification

NVMe queues and PCIe provide the required high-speed storage path, while DMA removes bulk data movement from the CPU. Interrupt notification is used only to report completion rather than transferring every byte.

# Q3. Multi-Device Communication Architecture

## Problem / Constraints

- 6 peripherals use USB.

- 2 high-speed devices use Thunderbolt.

- Interrupt starvation must be avoided.

- Hardware and software interrupts must be used.

- Fair bus arbitration is required.

## Proposed Solution

Use separate USB and Thunderbolt controllers connected through the system interconnect, with a priority-aware interrupt controller and fair arbitration such as round-robin or weighted round-robin.

## Architecture

CPU

↓

Interrupt Controller

■ ■

Hardware IRQs Software IRQs

↓ ↓

USB Host Controller Background Tasks

↓

U1 U2 U3 U4 U5 U6

Thunderbolt Controller

↓ ↓ T1 T2

## Working

USB peripherals communicate through the USB host controller, while the two high-speed devices use Thunderbolt.

Urgent events use hardware interrupts. Longer or background work is deferred to software interrupts.

Round-robin/weighted arbitration gives devices controlled access to shared resources.

## Anti-Starvation Measures

1. Assign priority levels. 2. Keep ISRs short. 3. Use interrupt moderation/coalescing where appropriate. 4. Use round-robin scheduling among devices of similar priority. 5. Defer lengthy work to software interrupts.

## Justification

Priority levels and interrupt moderation prevent high interrupt rates from monopolizing the CPU. Fair arbitration prevents one device from continuously occupying the bus and reduces starvation.

# Q4. 500 MB/s Data Acquisition System

## Problem / Constraints

- Continuous data stream of 500 MB/s.

- Minimal CPU intervention.

- Compare DMA transfer with interrupt-driven I/O.• Device registers must use memory-mapped I/O.

## Proposed Solution

Use DMA with double buffering and memory-mapped device registers. The CPU configures the DMA engine once, while data is transferred directly into memory.

## Architecture

Data Acquisition Device

↓ 500 MB/s

DMA Controller

■ ■

Buffer A Buffer B

■ ■

Main Memory

↓ CPU

## Working

The CPU configures the acquisition device and DMA registers through memory-mapped I/O. DMA fills Buffer A while the CPU processes Buffer B. When Buffer A is full, DMA switches to Buffer B and an interrupt notifies the CPU that the completed buffer is ready.

## DMA vs Interrupt-Driven I/O

DMA: very low CPU involvement, efficient for large continuous transfers, low interrupt overhead. Interrupt-driven I/O: requires frequent CPU service and becomes inefficient when interrupts are generated for many small data units. Choice:

DMA.

Final Design

Memory-mapped device registers + DMA controller + double buffering + buffer-completion interrupts.

## Justification

DMA is suitable for continuous high-volume transfer because the CPU does not handle every data item. Double buffering allows acquisition and processing to overlap, reducing the chance of data loss.

# Q5. Nested Interrupt Handling Mechanism

## Problem / Constraints

- Support nested interrupts.

- Maximum interrupt latency must be below 10 µs.

- Vectored interrupts are required for high-priority devices.

- Software interrupts are required for background tasks.

## Proposed Solution

Use a priority-based vectored interrupt controller with nested interrupt support. High-priority interrupts can preempt lower-priority ISRs, while background work is deferred through software interrupts.

## Architecture

High-Priority Devices

↓

Vectored Interrupt Controller

↓

CPU

■ ■

High ISR Low ISR

↓

Higher-priority IRQ

↓

Nested ISR

Background tasks

↓

Software Interrupts

## Working

When an interrupt occurs, the controller determines its priority and vector. The CPU saves the current context and enters the selected ISR. A higher-priority interrupt may preempt a lower-priority ISR. After the higher-priority ISR finishes, the saved context is restored and execution resumes.

## Example Priority Levels

Priority 3 – emergency/real-time device; Priority 2 – high-speed I/O; Priority 1 – normal I/O; Priority 0 – background software tasks.

## Latency Techniques

Use vectored interrupts, short ISRs, fast context saving, hardware priority logic, limited interrupt masking, and software interrupts for deferred processing. The <10 µs target should be verified on the selected hardware.

## Justification

Vectored interrupts reduce interrupt identification overhead, while priority-based nesting ensures critical devices receive rapid service. Short ISRs and deferred background processing help meet the <10 µs latency target.

# Summary of Final Designs

| Problem | Main Technique | Purpose |
| --- | --- | --- |
| Q1 | Memory-mapped I/O + vectored interrupts | Low latency and controlled CPU utilization |
| Q2 | NVMe + PCIe + DMA + interrupt notification | High-speed storage with low CPU overhead |
| Q3 | USB + Thunderbolt + fair arbitration | Prevent interrupt/bus starvation |
| Q4 | DMA + double buffering | Continuous 500 MB/s acquisition |
| Q5 | Nested vectored interrupts + software interrupts | Priority handling and <10 µs target |

Note: The solutions are based on the constraints and terminology stated in the uploaded assessment document.